# RGB LED Control V2.0 Design

Peter Essam | ARM | 21/11/2023

Table of Content

- Introduction

- High Level Design

- Layered Architecture

- Modules Descriptions

- Drivers Documentations

- GPIO

- SYSTICK

- SYSTICK MANAGER

- LED

- BUTTON

- Low Level Design

- Flow Chart

- Precompiling Configurations

- Linking Configurations

Introduction

- Overview:

This project is designed with a layered architecture, separating concerns into different layers for better maintainability and scalability. The project focuses on controlling an RGB LED using GPIO (General Purpose Input/Output) pins. The Microcontroller Abstraction Layer (MCAL) handles low-level hardware interactions, the Hardware Abstraction Layer (HAL) manages LED and button functionality, the Service Layer manage the drivers in MCAL to can included in application and the Common Layer provides standard library names for consistency.

- Layers:

MCAL (Microcontroller Abstraction Layer)

Responsible for low-level hardware interactions.

Utilizes GPIO to control hardware-level features.

Abstracts microcontroller-specific details.

HAL (Hardware Abstraction Layer)

Manages higher-level functionalities for LEDs and buttons.

Uses MCAL services to control GPIO pins.

Provides an abstraction for RGB LED control and button input.

Service Layer

Manage the drivers in MCAL layer to can included in App layer

Common Layer

Hosts standard library names and common services.

App (Application Layer)

This is the Application

- Project Functionality:

The main objective of this project is to control an RGB LED based on button presses and time calculated. The RGB LED is connected to specific GPIO pins on the microcontroller. When a button is pressed, the program detects the button press through the HAL layer, and the RGB LED changes its state accordingly and when the time finish the LED is off .

- Key Components :

MCAL Layer

GPIO driver: Provides low-level functions for GPIO pin initialization, reading, and writing.

HAL Layer

LED Interface: functions to control the RGB LED (e.g., turning on, turning off , ….).

Button Interface: Handles button-related operations (e.g., detecting button presses).

Service Layer

Facilitation control any driver in MCAL want to included in App layer.

Common Layer

Standard Library Names: Ensures consistent naming conventions and library usage across the project.

- Workflow:

Initialization:

MCAL initializes GPIO pins for the RGB LED.

HAL initializes LED and button components.

Button Press Detection:

HAL layer monitors the button state and detects button presses.

RGB LED Control:

Based on button presses, the HAL layer controls the RGB LED through the MCAL GPIO driver.

Possible actions: turn on, turn off, change color.

- Benefits:

Modularity

Each layer is modular, making it easier to modify or extend functionalities.

Abstraction

Higher layers abstract hardware details, promoting code readability.

Consistency

Standard library names in the common layer ensure consistent coding practices.

- Conclusion:

This project showcases a well-organized architecture with separate layers, each serving a specific purpose. The use of MCAL, HAL, Service Layer and a Common Layer contributes to code clarity, maintainability, and scalability, making it easier to manage and expand the functionality of the RGB LED control system.

High Level Design

- Layered Architecture

COMMON

APP

SERVICE (systick_Manager)

HAL (led, Button)

MCAL (DIO , SYSTICK)

- Modules Descriptions

MCAL Layer

GPIO driver : Provides low-level functions for GPIO pin initialization, reading, and writing to control  RGB Leds signals.

HAL Layer

LED driver: control the RGB LED (e.g., led initialization , led on, led off , led toggle)

Button driver: Handles button-related operations (e.g., button initialization, detecting button presses).\

Service Layer

Control MCAL drivers in App layer

Common Layer

Standard Library Names: to serve all project layers

App Layer

This is the application i want to do

- Drivers Documentations

- GPIO

(This Driver Located in MCAL Layer)

1-

enu_MGPIO_errorStatus_t MGPIO_init(str_MGPIO_configuration_t *ptr_str_MGPIO_config)

Description:

Initializes a GPIO pin based on the provided configuration.

Arguments:

ptr_str_MGPIO_config Pointer to a structure contain GPIO config

Return:

GPIO_OK: Successful initialization.

GPIO_NULL_POINTER: Null pointer argument.

GPIO_PORT_ERROR: Invalid port number.

GPIO_PIN_ERROR: Invalid pin number.

GPIO_DIRECTION_ERROR: Invalid pin direction.

GPIO_MODE_ERROR: Invalid mode selection.

GPIO_OUT_CURRENT_ERROR: Invalid output current.

GPIO_INTERNAL_TYPE_ERROR: Invalid internal type.

GPIO_VALUE_ERROR: Invalid output level.

2-

enu_MGPIO_errorStatus_t MGPIO_write(enu_MGPIO_portNumber_t enu_a_portNumber, enu_MGPIO_pinNumber_t enu_a_pinNumber, enu_MGPIO_pinValue_t enu_l_pinValue)

Description:

Write a value to a specific GPIO pin.

Arguments:

enu_a_portNumber  Select the GPIO port number.

enu_a_pinNumber   Select the GPIO pin number.

enu_l_pinValue    Select the value to be written to the pin (PIN_HIGH_VALUE or PIN_LOW_VALUE).

Return:

GPIO_OK                     Success operation.

GPIO_PORT_ERROR             Invalid port number.

GPIO_PIN_ERROR                  Invalid pin number.

GPIO_VALUE_ERROR            Invalid pin value.

GPIO_PORT_NOT_INITIALIZED   Port not initialized.

3-

enu_MGPIO_errorStatus_t MGPIO_read(enu_MGPIO_portNumber_t enu_a_portNumber, enu_MGPIO_pinNumber_t enu_a_pinNumber, boolean *ptr_arg_pinValue)

Description:

Read the value of a specific GPIO pin.

Arguments:

enu_a_portNumber    Select the GPIO port number.

enu_a_pinNumber     Select the GPIO pin number.

ptr_arg_pinValue        Pointer to a boolean variable to store the read value.

Return

GPIO_OK                                           Success operation.

GPIO_PORT_ERROR          	         Invalid port number.

GPIO_PIN_ERROR                            Invalid pin number.

GPIO_NULL_POINTER                     Null pointer argument.

GPIO_PORT_NOT_INITIALIZED    Port not initialized.

4-

enu_MGPIO_errorStatus_t MGPIO_read(enu_MGPIO_portNumber_t enu_a_portNumber, enu_MGPIO_pinNumber_t enu_a_pinNumber, boolean *ptr_arg_pinValue)

Description:

Toggle the value of a specific GPIO pin.

Arguments:

enu_a_portNumber    Select the GPIO port number.

enu_a_pinNumber     Select the GPIO pin number.

Return

GPIO_OK                                           Success operation.

GPIO_PORT_ERROR          	         Invalid port number.

GPIO_PIN_ERROR                            Invalid pin number.

GPIO_PORT_NOT_INITIALIZED    Port not initialized

- SYSTICK

(This Driver Located in MCAL Layer)

1-

enu_SysTick_Error_t SysTick_Init(uint32_t reload_value, enu_SysTick_ClockSource_t clk_source)

Description:

Initialize the SysTick timer.

Arguments:

reload_value: The value to load into the SysTick Reload register.

param clk_source: The clock source for SysTick

Return:

enu_SysTick_Error_t: Error status after initialization.

2-

enu_SysTick_Error_t SysTick_Start(void)

Description:

Start the SysTick timer.

Return:

enu_SysTick_Error_t: Error status after start the Systick.

3-

enu_SysTick_Error_t SysTick_Stop(void)

Description:

Stop the SysTick timer.

Return:

enu_SysTick_Error_t: Error status after stop the Systick.

4-

enu_SysTick_Error_t SysTick_DelayMs(uint32_t delay_ms)

Description:

Delay the execution for a specified number of milliseconds using SysTick

Arguments:

The delay time in milliseconds.

Return:

enu_SysTick_Error_t: Error status after delay.

5-

enu_SysTick_Error_t SysTick_DelayUs(uint32_t delay_us)

Description:

Delay the execution for a specified number of microseconds using SysTick

Arguments:

The delay time in microseconds.

Return:

enu_SysTick_Error_t: Error status after delay.

6-

uint8_t SysTick_CheckTimeOut(void)

Description:

Check Systick timer reached Zero

Return:

1 if the COUNTFLAG is set, indicating a timeout; otherwise, 0.

- LED

(This Driver Located in HAL Layer)

1-

enu_ledErrorState_t H_LED_init(enu_MGPIO_portNumber_t enu_l_ledPort, enu_MGPIO_pinNumber_t enu_l_ledPin)

Description:

Initialize a LED on a specific GPIO port and pin

Arguments:

enu_l_ledPort    Select the GPIO port number for the LED.

enu_l_ledPin     Select the GPIO pin number for the LED.

Return

LED_OK                   Success initialization.

LED_NOT_OK          LED initialization not successful.

2-

enu_ledErrorState_t H_LED_on(enu_MGPIO_portNumber_t enu_l_ledPort, enu_MGPIO_pinNumber_t enu_l_ledPin)

Description:

Turn on a LED connected to a specific GPIO port and pin.

Arguments:

enu_l_ledPort    Select the GPIO port number for the LED.

enu_l_ledPin     Select the GPIO pin number for the LED.

Return

LED_OK                    Success initialization.

LED_NOT_OK          LED initialization not successful.

3-

enu_ledErrorState_t H_LED_off(enu_MGPIO_portNumber_t enu_l_ledPort, enu_MGPIO_pinNumber_t enu_l_ledPin)

Description:

Turn off a LED connected to a specific GPIO port and pin.

Arguments:

enu_l_ledPort    Select the GPIO port number for the LED.

enu_l_ledPin     Select the GPIO pin number for the LED.

Return

LED_OK                    Success initialization.

LED_NOT_OK          LED initialization not successful.

4-

enu_ledErrorState_t H_LED_toggle(enu_MGPIO_portNumber_t enu_l_ledPort, enu_MGPIO_pinNumber_t enu_l_ledPin)

Description:

Toggle the state of an LED connected to a specific GPIO port and pin.

Arguments:

enu_l_ledPort    Select the GPIO port number for the LED.

enu_l_ledPin     Select the GPIO pin number for the LED.

Return

LED_OK                    Success initialization.

LED_NOT_OK          LED initialization not successful.

- BUTTON

(This Driver Located in HAL Layer)

1-

enu_buttonErrorStatus_t H_BUTTON_init(void)

Description:

Initialize the configuration of all buttons.

Return:

BUTTON_OK              Success initializing all buttons.

BUTTON_NOT_OK   Failed to initialize buttons.

2-

enu_buttonErrorStatus_t H_BUTTON_read(enu_buttonNumber_t enu_a_button_Number, boolean *ptr_a_value)

Description:

Read the state of a specific button.

Arguments:

enu_a_button_Number  The button number to read.

ptr_a_value          Pointer to a boolean variable to store the button state.

Return:

BUTTON_OK              Success initializing all buttons.

BUTTON_NOT_OK   Failed to initialize buttons.

- SYSTICK_MANAGER

(This Driver Located in Service Layer)

1-

enu_SysTickManager_Error_t SysTickManager_Init(uint32_t reload_value, enu_SysTick_ClockSource_t clk_source)

Description:

Initialize the SysTick timer .

Arguments:

reload_value: The reload value for the SysTick timer.

clk_source: The clock source for the SysTick timer.

Return

enu_SysTickManager_Error_t: Error status.

2-

enu_SysTickManager_Error_t SysTickManager_Start(void)

Description:

Start the SysTick timer .

Return

enu_SysTickManager_Error_t: Error status.

3-

enu_SysTickManager_Error_t SysTickManager_Stop(void)

Description:

Stop the SysTick timer .

Return

enu_SysTickManager_Error_t: Error status.

4-

enu_SysTickManager_Error_t SysTickManager_DelayMs(uint32_t delay_ms)

Description:

Dleay the SysTick timer .

Arguments:

delay_ms: The value in millisec for the SysTick timer want delay.

Return

enu_SysTickManager_Error_t: Error status.

5-

enu_SysTickManager_Error_t SysTickManager_DelayUs(uint32_t delay_us)

Description:

Dleay the SysTick timer .

Arguments:

delay_us: The value in microsec for the SysTick timer want delay.

Return

enu_SysTickManager_Error_t: Error status.

Low Level Design

- Flow Chart

- GPIO

enu_MGPIO_errorStatus_t MGPIO_write(enu_MGPIO_portNumber_t enu_a_portNumber, enu_MGPIO_pinNumber_t enu_a_pinNumber, enu_MGPIO_pinValue_t enu_l_pinValue)

enu_MGPIO_errorStatus_t MGPIO_read(enu_MGPIO_portNumber_t enu_a_portNumber, enu_MGPIO_pinNumber_t enu_a_pinNumber, boolean *ptr_arg_pinValue)

enu_MGPIO_errorStatus_t MGPIO_toggle(enu_MGPIO_portNumber_t enu_a_portNumber , enu_MGPIO_pinNumber_t enu_a_pinNumber)

- SYSTICK

enu_SysTick_Error_t SysTick_Init(uint32_t reload_value, enu_SysTick_ClockSource_t clk_source)

enu_SysTick_Error_t SysTick_Start(void)

enu_SysTick_Error_t SysTick_Stop(void)

enu_SysTick_Error_t SysTick_DelayMs(uint32_t delay_ms)

enu_SysTick_Error_t SysTick_DelayUs(uint32_t delay_us)

uint8_t SysTick_CheckTimeOut(void)

- LED

enu_ledErrorState_t H_LED_init(enu_MGPIO_portNumber_t enu_l_ledPort , enu_MGPIO_pinNumber_t enu_l_ledPin)

enu_ledErrorState_t H_LED_on(enu_MGPIO_portNumber_t enu_l_ledPort , enu_MGPIO_pinNumber_t enu_l_ledPin)

enu_ledErrorState_t H_LED_off(enu_MGPIO_portNumber_t enu_l_ledPort , enu_MGPIO_pinNumber_t enu_l_ledPin)

enu_ledErrorState_t H_LED_toggle(enu_MGPIO_portNumber_t enu_l_ledPort , enu_MGPIO_pinNumber_t enu_l_ledPin)

- BUTTON

enu_buttonErrorStatus_t H_BUTTON_init(void)

enu_buttonErrorStatus_t H_BUTTON_read(enu_buttonNumber_t enu_a_button_Number , boolean *ptr_a_value)

- APP

void app_init(void)

void app_Start(void)

- Precompiling & Linking Configurations

- GPIO

- SYSTICK

- LED

- BUTTON

That is all requirements

Thanks